Step To Do: Assigned to new APC

Since you're already doing APC, I'd suggest thinking about every new APC application in 3 stages: Understand → Analyze → Improve.

Interestingly, this aligns very closely with PETRONAS APC implementation practices documented in 164301 Advanced Process Control v1 and AMENDED PTS APC 16.43.01 (Revised - 11 November 24) 1, which emphasize Readiness Assessment, BLCA, Feasibility Study, FDS, Step Testing, Model Identification, Controller Building, Commissioning, and Continuous Performance Monitoring. [164301 Adv...Control v1 | Word], [AMENDED PT...mber 24) 1 | Word]

1. UNDERSTAND THE PROCESS FIRST

Before touching the APC configuration:

Process Understanding

- Review PFD, P&ID, operating manuals.

- Understand process objectives:

  - Throughput maximization?

  - Energy minimization?

  - Quality control?

  - Constraint management?

- Identify:

  - MV (Manipulated Variables)

  - CV (Controlled Variables)

  - DV (Disturbance Variables)

Operations Understanding

- Interview operators.

- Ask:

  - What are the major operating challenges?

  - What limits production?

  - Which variables operators watch most?

  - What actions do they frequently take manually?

APC Strategy Understanding

Review:

- FDS (Functional Design Specification)

- Previous APC reports

- Benefits analysis

- Controller configuration

Ask yourself:

Why was this APC built?

Many engineers understand "how" the controller works but not "why" it exists.

2. ANALYZE THE EXISTING APC

APC Health Check

Utilization

Check:

- APC ON percentage

- APC OFF reasons

- MVs detached?

- CVs inactive?

Example:

| Parameter | Good |
| --- | --- |
| APC uptime | >80% |
| CV in control | >90% |
| MV available | >90% |

Constraint Analysis

Identify:

- Which CV hits limits most?

- Which MV saturates?

- What is limiting production?

Often 80% of APC value comes from understanding the active constraints.

Controller Performance

Review:

- CV standard deviation

- Constraint violations

- Settling time

- Oscillation

- APC economic objective achievement

BLCA assessment guidelines in your feasibility studies classify controllers based on uptime, compliance, noise, settling behavior, and tuning quality. [PCLDPE APC...y Study_V1 | Word], [PCLDPE APC...lity Study | Word]

Model Quality

Check:

- Gain

- Deadtime

- Time constant

- Prediction accuracy

Questions:

- Does model prediction match plant response?

- Has process changed since commissioning?

- Was equipment modified?

Red flags:

- Large model mismatch

- Frequent MV moves

- Poor CV tracking

3. ANALYZE THE ECONOMICS

This is where senior APC engineers differentiate themselves.

Instead of asking:

Is APC working?

Ask:

Is APC creating value?

Determine:

Production

- Additional feed

- Additional throughput

Energy

- Steam reduction

- Fuel reduction

- Power reduction

Quality

- Less giveaway

- Better specification control

Reliability

- Fewer trips

- Smoother operation

4. FIND IMPROVEMENT OPPORTUNITIES

Control Layer

Review:

Poorly Tuned Base Layer Loops

Many APC issues are actually PID issues.

Check:

- Oscillation

- Sluggish response

- Valve stiction

- Sensor noise

As PETRONAS APC standards highlight, BLCA findings and corrective actions should be completed before APC optimization. [164301 Adv...Control v1 | Word], [AMENDED PT...mber 24) 1 | Word]

New MVs

Ask:

Can I add more degrees of freedom?

Examples:

- Reboiler duty

- Reflux flow

- Compressor guide vane

- Recycle valve

New CVs

Ask:

What important variable is currently unmanaged?

Examples:

- Product quality

- Energy index

- Constraint variable

- Equipment loading

Inferential Enhancement

Can laboratory measurements be replaced by:

- Soft sensor

- Inferential model

- Estimator

This often unlocks additional APC value.

5. VALIDATE THROUGH DATA

Never rely solely on operator feedback.

Perform:

Trend Analysis

- 1 week

- 1 month

- Normal operation

- Disturbed operation

Step Response Analysis

Confirm:

- Gain

- Deadtime

- Dynamic response

Benefit Assessment

Compare:

- APC ON

- APC OFF

Metrics:

- Throughput

- Energy

- Quality

- Variability

6. DEVELOP A SUSTAINABILITY PLAN

The best APC is not the one that performs today.

It is the APC that still performs 3 years later.

Check:

- Operator acceptance

- Documentation quality

- APC maintenance procedure

- Model update strategy

- Performance monitoring dashboard

Continuous APC performance monitoring and post-commissioning reviews are recommended as part of the APC lifecycle. [164301 Adv...Control v1 | Word], [AMENDED PT...mber 24) 1 | Word]

My Personal APC Workflow (What I Would Do on Day 1)

- Read PFD, P&ID, FDS.

- Understand business objective.

- Review APC architecture.

- Identify MV, CV, DV.

- Analyze APC utilization.

- Analyze active constraints.

- Analyze APC benefits.

- Review model quality.

- Review BLCA findings.

- Meet operators.

- Generate improvement opportunities.

- Rank opportunities by value vs effort.

Golden Question

Whenever assigned a new APC application, always ask:

"What is currently limiting the plant from achieving maximum economic performance, and how can APC remove that limitation?"

That single question will guide your process understanding, controller analysis, and improvement strategy.